OpenJazz CD32X – Project Summary

Oct 7, 2026 · @erexx

At a glance

Jazz Jackrabbit (1994, Epic MegaGames) runs on a real Sega CD + 32X with MicroNut99's 4 MB RAM cart, built on the OpenJazz engine: all 79 levels of the CD edition, CD-audio music, PCM sound effects, saves that survive a power cycle, and a quick start on the second boot. Port by Micronut99; current build J14 (frame profiler on).

The game is playable start to finish. The open problem is speed: levels run at about 11 frames per second, so scrolling is not smooth. Bonus levels, cutscenes and Holiday Hare '95 are not in yet.

How it works

Four processors share the work; the SH2s run the game, the Sega CD side does everything the SH2s cannot reach.

| Processor | Runs | Job |
| --- | --- | --- |
| 32X master SH2 | the OpenJazz engine (C++, from the cart) | game logic, drawing straight into the 32X frame buffer |
| 32X slave SH2 | same program | draws half of the screen in parallel (tile layers, credits plasma) |
| Sega CD Sub-CPU | OpenLara's Sub-CPU program | reads the disc, copies data into the cart, plays CD audio and PCM effects, writes saves |
| Genesis 68000 | Chilly Willy's listener | 32X registers, pad, cart writes for the Sub-CPU |

The RAM cart is the main memory. The SH2 can read it but not write it, so the PC build prepares everything: an engine image, a 1.4 MB common pack (menus, fonts, panel, demos) and 39 world packs (both levels of a planet, tiles, sprites, planet screen, all pictures already decoded). The engine copies the pack it needs into the cart with the Sub-CPU's commands and reads it in place. Saves live in a 64 KB area at the end of the cart.

Audio stays on the Sega CD side. Music is 35 CD audio tracks rendered from the game's own modules (one complete pass each, repeated by the drive). The 22 sound effects sit in the PCM chip's 64 KB and are triggered with the pad request.

Booting uses OpenLara's proven loader unchanged except for marked switches; a quick check skips copying what is already in the cart (A+B+C at power-on forces a full load).

Features and state

Everything below was tested on the console unless noted.

| Feature | State | Notes |
| --- | --- | --- |
| All 79 levels, 9 episodes | works | one pack per planet; second level of a planet starts without loading |
| Menus, set-up, demo mode | works | demo after 20 s at the main menu; set-up is pad-only (A/B swap, music and effects on/off) |
| Music | works | 35 CD audio tracks, each song one complete pass, repeated |
| Sound effects | works | 22 effects on the PCM chip, filtered at 5 kHz |
| Saves and settings | works | RAM cart save area, survive power-off |
| Quick start | works | second boot in seconds; A+B+C forces a full load |
| Credits and CD player | works | plasma background, selector separate from playback, endless playlist, Y/X/Z/Mode or Up/Down hide the text |
| Frame rate in levels | limited | about 11 fps; J14 shows where the frame time goes |
| Bonus levels | not yet | skipped: their map needs 128 KB of SDRAM |
| Cutscenes and intro | off | planned as pre-rendered cells |
| Holiday Hare '95 | not yet | a second data set with the same file names |
| Emulators | limited | needs the 4 MB RAM cart; Fusion stops on the splash |

Key numbers

| Measure | Value |
| --- | --- |
| SH2 engine (code + data) | 417 KB, runs from the cart |
| SDRAM heap for the game | about 218 KB of 256 KB |
| Level memory peak (PC, 32X code paths, incl. a 64 KB screen the console does not keep) | at most 257 KB → about 193 KB on the console |
| Common pack | 1.43 MB (cart 0x100000) |
| World packs | 39 packs, 34 MB in all, largest 1.49 MB (cart 0x280000) |
| Disc | about 61 minutes: 64 MB data + 53 minutes of music |
| Loading speed | about 30 KB/s (~1 s per 32 KB chunk) |
| Frame rate in levels | about 11 fps (before J14 profiling) |
| PC regression | 706 frames, pixel-identical after every change |

Next steps and credits

- Read the J14 profiler on the console (time per frame part) and fix the biggest cost first.

- Line-table scrolling renderer behind OJ32X_SCROLL: the level stays in the frame buffer, the line table moves it, only new tile strips and the sprites are drawn; the sky is kept right by rotating its palette.

- Faster loading: bigger chunks per command, compression unpacked by the Sub-CPU, preloading the next planet.

- Bonus levels, Holiday Hare '95, cutscenes as pre-rendered cells.

Credits. Jazz Jackrabbit: Epic MegaGames (Arjan Brussee, Cliff Bleszinski). OpenJazz: Alister Thomson and contributors. Sega CD + 32X framework: Chilly Willy. D32XR 32X code: Victor Luchits. Boot path from OpenLara's CD32X port. Sega CD32X port: Micronut99.

Source and documents. The source release (OpenJazzCD32X-src.zip) holds the engine changes, port layer, boot programs and tools, without any game data; OPENJAZZ_CD32X_HANDOFF.md is the round-by-round log, and OpenJazz CD32X – Addendum to the Kobo CD32X Boot Documents covers the technical lessons.